ECEN 340 Midterm Exam – Lab

Objective:  Write and implement the Verilog code for the Gray-code counter you designed in the written portion of the exam (the sequence is 111, 011, 010, 110, 100, 000, 001, 101).  There will be four modules required to implement the design.

Module 1, “gray_counter”

This module will implement the gray code counter.  When implementing the counter, use the following structure:

- All three flip-flops will be implemented with one “always” block.  Use one of the push-button switches to reset the flip-flops. This always block will contain ONLY the flip-flops.

- The inputs to the flip-flops will be the “next state”  (or input forming) logic. The input forming logic will be designed using a separate “always” statement (procedural format), or by using an “assign” statement for each next_state variable (data flow format).

- The count will be displayed on 3 LEDs, and the count will increment slow enough to be seen and verified (in your report you will discuss the level of functionality of your simulation and the level of functionality of the implemented design).

Module 2, “clock_gen”

This module will be nearly identical to the clock generator you designed in Lab 6, but you will use the slowest clock frequency available (output 25) so that the codes may be viewed as they sequence from one count to the next on the LED outputs.  You may refer to your Lab 6 or code for this implementation.

It will be incredibly difficult to simulate a clock that is this slow, so you may temporatily set the clock frequency to a faster output for simulation.

Note: If you were not successful at getting the Lab 6 generator to function, you may consult with the instructor on how to do this, but you may not consult with another student!

Module 3, “gray_counter_top”

This module will simply instantiate Modules 1 and 2, and will contain the ports for the 100MHz clock, the reset signal, and the 3 LEDs as outputs.

Module 4 “gray_counter_top_tb”

This will be the test bench.  It will instantiate the “gray_counter_top” module, and will be used to generate the 100 MHz clock and the reset signal for the flip-flops for simulation purposes.  You should also place a separate “initial #xxxxxx $finish” statement in this module, where xxxxxx represents the expected duration of the simulation.

As mentioned in the “clock_gen” description, it is a good idea to temporarily set the clock_gen output to a faster clock so that simulation will be faster.  Restore the clock to the slowest clock when you actually implement the design.

Report Requirements

1) Include title page, procedures, and the source code for all four modules (25 pts).

2) Simulate your gray counter in Vivado and take a snapshot of the generated clock and the count pattern, and insert this in your report (25 pts).

3) Describe the level of functionality of your design (25 pts).

4) Think of a simpler or alternative method to implement this counter in Verilog, and describe this method (25 pts).

The following notes are to help you complete the design. Refer to them as needed:

For your convenience, below is the replicaion of the trasition table and the K-maps for the Gray counter from the written part of the exam:

Gray[2:0]| Gray_next[2:0]

_________|_______________

0 0 0    |   0 0 1

0 0 1    |   1 0 1

0 1 0    |   1 1 0

0 1 1    |   0 1 0

1 0 0    |   0 0 0

1 0 1    |   1 1 1

1 1 0    |   1 0 0

1 1 1    |   0 1 1

Gray[1:0] \ Gray[2]

\__0___1__

0 0 | 0 | 0 |

0 1 | 1 | 1 |

1 1 | 0 | 0 |

1 0 | 1 | 1 |

Gray_next[2] = Gray[1]’*Gray[0] + Gray[1]*Gray[0]’

Gray[1:0] \ Gray[2]

\__0___1__

0 0 | 0 | 0 |

0 1 | 0 | 1 |

1 1 | 1 | 1 |

1 0 | 1 | 0 |

Gray_next[1] = Gray[2]’*Gray[1] + Gray[2]*Gray[0]

Gray[1:0] \ Gray[2]

\__0___1__

0 0 | 1 | 0 |

0 1 | 1 | 1 |

1 1 | 0 | 1 |

1 0 | 0 | 0 |

Gray_next[0] = Gray[2]’*Gray[1]’ + Gray[2]*Gray[0]

_____

Gray_next[2]-|D   Q|- Gray[2]

|     |

|     |

|>    |

_____

Gray_next[1]-|D   Q|- Gray[1]

|     |

|     |

|>    |

_____

Gray_next[0]-|D   Q|- Gray[0]

|     |

|     |

|>    |