Additional BJT Circuits HW (25 points)

(jas, Additional BJT Circuits HW.docx, 7/26/2025)

Clearly indicate answers and include units for numeric answers, using 3 or 4 significant figures. Show at least the major steps of your work so that if necessary partial credit can be awarded.

- Determine the following values for the adjacent common-collector (emitter-follower) circuit with a dc bias voltage of 6.1 V at Vin. (Hint: AV should be somewhere between 0.95 and 0.99.) (8 points total.)

- re1   (2 Points.)

re=RE∥RL=2.7kΩ∥360Ω=317.6Ω

re=317.5Ω

- re1'   (2 Points.)

re'=25mVIE

IE=Vin-0.7VRE=6.1-0.7V2.7kΩ=2.0mA

re'=25mV2.0mA=12.5Ω

re'=12.5Ω

- Zin   (2 Points.)

Zinbase=βre+re'=110317.6Ω+12.5Ω=36316Ω=36.3kΩ

Zinbase=36.3kΩ

- AV   (2 Points.)

Av=rere+re'=110317.6Ω+12.5Ω=0.962

Av=0.962

- Determine the following values for the adjacent cascaded common-emitter and common-collector (emitter follower) amplifier circuit, given the voltage gain of -144 for the common-emitter stage with voltage VC1 = 5.9 V. (Hint: The magnitude of AV_Total should be between 132 and 142.) Be sure to include units in your answers. (8 points total.)

- re2. (2 Points.)

re2=RE2∥RL=1.5kΩ*150Ω1.5kΩ+150Ω=136.36Ω

re2=136.36 Ω

- re2'. (2 Points.)

re2'=25mV3.47mA=7.21Ω

re'=7.21Ω

- Voltage gain of the common-collector (emitter follower) stage AVCC. (2 Points.)

AVCC=rere+re'=136.36136.36+7.21=0.9498

AVCC=0.9498

- Overall voltage gain AV_Total. (2 Points.)

AVTotal=-1440.9498=-136.77

AV_Total =-136.77

- For the adjacent Darlington circuit, determine the current Iin, given β = 120 for both Q1 and Q2.

(2 points.)

VB1=-8 V

VB2=VB1-0.7 V

Vout=VB1-1.4 V

IRload=VoutRload

IC2=β2IB2=IRload

IB2=IE1≈IC1

IC1=β1IB1

IB1=IC1β1

VB2=-8.7 V

Vout= -9.4 V

IRload=-9.4V10Ω=-0.94 A

-0.94 A=120*IB2

IB2=-0.0078 A= -7.8mA

IE1≈IC1=-7.8mA
-7.8 mA=120*IB1

IB1= -65.28 μA=Iin

Iin=-65.28 μA

- Calculate the following values for the adjacent voltage regulator circuit with a 10 Ω load. (Hint: The output current of op-amp U1 is between 65 and 75 µA.) (7 points total.)

- Vout. Document your work so that others can follow your method, including assumptions along with at least 2 steps involved in arriving at Vout. A correct answer without any work shown and/or mention of assumptions will only receive 1 out of 3 points. (3 points.)

Vp=Vn=VR2=4.0 V

IR2=IR1=4.0V20kΩ=0.2 mA

VR1=IR1*R1=2V

Vout=VR1+VR2=2V+4V=6V

Vout=6V

- IRload. (2 points.)

IRload=VoutRload=0.6A

IRload=0.6A

- Iout of op-amp U1. (2 points.)

IC2≈IE2=IRload+IR1=600.2 mA

IB2=IC2β2=5.456 mA

IC1≈IE1=IB2=5.456 mA

IoutU1=IB1=IC1β1=68.201 μA

IoutU1=68.201 μA