Bipolar Junction Calculation HW (18 Points)

(jas, BJT Calculation HW.docx, 2/26/2024)

- Determine the base current IB of Q1 in the adjacent circuit, given VBE = 0.7 V. (Hint: Your answer should be between 20 and 30 μA.) (1 point.)

IB=3-0.7 V100*103Ω=23 μA

IB=23 μA

v

- Determine the collector current IC of Q1 in the adjacent circuit given VBE = 0.7 V and β = 100. (Hint: Your answer should be between 1.8 and 2.2 mA.)

(2 points.)

IB=2.7-0.7 V100*103Ω=20 μA

IC=βIB=10020 μA=2000 μA=2.0 mA

IC=2.0 mA

- Determine the following for the adjacent circuit given β = 53 for Q1. (Hint: IB should be between 40 and 50 μA.) (3 points total.)

- Vout. (2 points.)

IB=3-0.7 V51*103Ω=45 μA

IC=5345 μA=2.39 mA

VCE=Vo=VCC-VR2=5V-2.39 mA2kΩ=0.22 V

Vout=0.22 V

- The region of operation for Q1, with the choices being Cutoff, Circuit Saturation, or the Active Region.  (1 point.)

VCE<0.3V→cuttoff

- Determine the following for the adjacent circuit given β = 100 for Q1. (Hint: IB should be between 25 and 35 μA.) (3 points total.)

- Vout. (2 points.)

IB=3.7-0.7 V100*103Ω=30 μA

IC=10030 μA=3.0 mA

VR2=3mA*1kΩ

VBE=Vout=VCC-VR2=2 V

Vout=2 V

- The region of operation for Q1, with the choices being Cutoff, Circuit Saturation, or the Active Region. (1 point.)

VCE=2 V>0.7 V→Active

- Determine the base current IB of Q1 in the adjacent circuit given VEB = 0.7 V and noting that conventional current is considered negative when flowing out of a transistor terminal. (Hint: Your answer should have a magnitude between 40 and 50 μA.) (1 point.)

IB=VCC-VEBR1=5-0.7100 kΩ=43 μA

IB=-43 μA (flowing out of base)

- Determine the collector current IC of Q1 in the adjacent circuit given VBE = 0.7 V, β = 110, and noting that conventional current is considered negative when flowing out of a transistor terminal. (Hint: Your answer should have a magnitude between 4.2 and 4.6 mA.) (2 points.)

IB=VCC-VBER1=4.7-0.7 V100*103Ω=40 μA

IC=βIB=11040 μA=4.4 mA

IC=4.4 mA

- Determine the following for the adjacent circuit given β = 95 for Q1. (Hint: ∣IB∣ should be between 18 and 25 μA.) (3 points total.)

- Vout. (2 points.)

VBE=0.7 V

IB=5-0.7 V200*103 Ω=21.5 μA

IC=βIB=9521.5 μA=2.04 mA

Vo=ICR2=2.04 mA1 kΩ=2.04 V

Vo=2.04 V

- The region of operation for Q1, with the choices being Cutoff, Circuit Saturation, or the Active Region. (1 point.)

VCE=VCC-VO=5V-2.04V=2.96V→active region

- Determine the following for the adjacent circuit given β = 60 for Q1. (Hint: ∣IB∣ should be between 30 and 50 μA.) (3 points total.)

- Vout. (2 points.)

VBE=0.7 V

IB=4.7-0.7 V100 kΩ=40 μA

IC=βIB=6040 μA=2.4 mA

Vo=ICR2-2.3 mA 1.91 kΩ=4.58 V

Vo=4.58 V

- The region of operation for Q1, with the choices being Cutoff, Circuit Saturation, or the Active Region. (1 point.)

VCE=5V-4.58 V=0.416 V>0.3→active