Differential Amplifier HW (19 points)

(jas, Differential Amplifier HW.docx, 12/8/2025)

Clearly indicate answers and include units for numeric answers, using 3 or 4 significant figures. Show at least the major steps of your work so that if necessary partial credit can be awarded.

- For the adjacent current-mirror circuit, given that Q1 and Q2 are matched transistors operating in the active region, calculate the value of R1 necessary to set the mirrored current, i.e. IR2, to 100 µA. (Hint: R1 should be somewhere between 100 and 120 kΩ.) (2 points.)

R1=12V-0.7V100μA=113kΩ

R1=113kΩ

- For the adjacent differential amplifier, determine the following, given that Q1 – Q4 are matched transistors operating in the active region. (Hint: IQ1 should be somewhere between 1.0 and 1.2 mA.) (3 points total.)

- VE.   (1 Point.)

VE=VBE=-0.7V

VE=-0.7V

- IQ1.   (2 Points.)

IQ4=11.93V--1210kΩ=2.393mA=IQ3

IQ1=IQ32=1.1965mA

IQ1=1.1965mA

- For the adjacent differential amplifier with β = 150 for all transistors, determine the following, given that Q1 – Q6 are matched transistors operating in the active region. (Hint: IB1 should be somewhere between 10 and 20 nA.) (5 points total.)

- VE.   (1 Point.)

VE=0.7V+0.7V=1.4V

VE=1.4V

- IQ5.   (2 Points.)

IQ6=12--11.336kΩ=647μA=IQ5

IQ5=647μA

- IB1.   (2 Points.)

IB1=ICQ52β2=647μA2*1502=14.38nA

IB1=14.38nA

- For the adjacent differential amplifier, determine the following for RL = ∞, given that Q1 - Q4 are matched transistors operating in the active region. (Hint: AV_dB should be somewhere between 40 and 46 dB.) (6 points total.)

- IQ3.   (2 Points.)

10V--9.3V12kΩ=1.608mA=IQ3

IQ3=1.608mA

- IQ2.   (1 Point.)

IQ2=IQ32=804μA

IQ2=804μA

- re2'.   (1 Point.)

re2'=25mV804μA=31.09Ω

re2'=31.09Ω

- AV_dB. (2 Points.)

Av=RC2re'=10kΩ231.09Ω=160.83

Av_dB=20log160.83=44.13dB

Av_dB=44.13dB

- Determine the open-loop gain of a differential amplifier with a common-mode rejection ratio of 86 dB and a common-mode gain of 30 x 10-3. (Hint: AV should be somewhere between 550 and 650.) (3 points.)

CMRRdB=20logAVACM

AV=10CMRRdB20*ACM=208620*30*10-3=598.58

AV=598.58