MOSFET HW 2 (16 points)

(jas, MOSFET HW 2.docx, 11/25/2025)

Clearly indicate answers and include units for numeric answers, using 3 or 4 significant figures. Show at least the major steps of your work so that if necessary partial credit can be awarded.

- The data sheet for the n-channel MOSFET in the adjacent circuit provides the following values: VT = 2.0 V, with ID = 0.5 A for VGS = 5.0 V and VDS = 10 V. (8 points total.)

- Using Sah’s equations and the data sheet values, determine βn for the MOSFET. (3 points.)

Region of operation?

VGS-VT=3.0V

5.0V≥3.0V→Transistor Saturation

ID=βn2VGS-VT2

βn=2IDVGS-VT2=20.5A5.0V-2.0V2=0.111AV2

βn=0.111AV2

- Using Sah’s equations and your previously calculated βn value, determine the drain current ID for the                                                                                                                                                                                                                                                                   MOSFET in the above circuit, including units, given that Vout = 0.2 V. (3 points.

ID=0.111AV25.0V-2.0V0.2V-0.2V22=0.0644 A

ID=0.0644 A

- Using your previously calculated ID value for Vout = 0.2 V in the above MOSFET circuit, determine RDS(ON)=VDSID, including units. (Hint: RDS(ON) should be in the range of 2.5 to 3.5 Ω.) (2 points.)

RDS(ON)=0.2V0.0644A=3.103Ω; RDS(ON)=3.103Ω

- The data sheet for the p-channel MOSFET in the adjacent circuit provides the following values: VT = -2.0 V, with ID = -0.24 A for VGS = -5.0 V and VDS = -10 V, which is the same as VSG = 5.0 V and VSD = 10 V. (8 points total.)

- Using Sah’s equations and the data sheet values, determine βp for the MOSFET. (3 points.)

Region of operation?

VSD≥VSG-VT→Transistor Saturation

ID=βp2VSG-VT2

βp=-2IDVSG-VT2=-2-0.24A5.0V--2.4V2=0.0533AV2

βp=0.0533AV2

- Using Sah’s equations and your previously calculated βp value, determine the drain current ID for the MOSFET in the above circuit, including units, given that Vout = 4.8 V. (3 points.)

Region of operation?

Vdd-Vout=VSD→5.0V-4.8V=0.2V

VSG≥VT;0<VSD<VSG-VT→Ohmic

ID=-βpVSG-VTVSD-VSD22=-0.0533AV25.0V--2.0V0.2V-0.222

ID=-0.0309 A

- Using your previously calculated ID value for Vout = 4.8 V in the above MOSFET circuit, determine RDS(ON)=VSD|ID|, including units. (Hint: RDS(ON) should be in the range of 6.3 to 6.6 Ω.) (2 points.)

RDSON=VSD|ID|=0.2V-0.0309A=6.4655 Ω

RDS(ON)=6.4655 Ω