MOSFET HW 3 (19 points)

(jas, MOSFET HW 3.docx, 12/1/2025)

Clearly indicate answers and include units for numeric answers, using 3 or 4 significant figures. Show at least the major steps of your work so that if necessary partial credit can be awarded.

- Determine the voltage Vin required to produce a drain current of 0.5 A in the adjacent enhancement mode MOSFET circuit, given VT = 2.0 V, and βn = 120 mA/V2.

(3 Points.)

ID=βn2VGS-VT2

VGS=Vin=2IDβn+VT=20.5A120*10-3A/V2+2.0V=4.8868V

Vin=4.8868 V

- The AOTFL296L n-channel enhancement mode power MOSFET has a typical threshold voltage VT of 2.9 V and a drain current of 66 A for VDS = VGS = 5 V.

- Using Sah’s equation with the substitution βn=μnCoxWL, determine βn for this power MOSFET. (2 Points.)

ID=βn2VGS-VT2

βn=2IDVGS-VT2=266A5V-2.9V2=29.932AV2

βn=29.932AV2

- Sah’s equation for the Ohmic region for an n-channel enhancement mode MOSFET is

as follows:ID=βnVGS-VTVDS-VDS22. Derive an expression for RDSOn=VDSID

in the Ohmic region in terms of βn, VGS, and VT from the above equation by neglecting the VDS22 term, which becomes negligible at small VDS voltages.

(2 Points.)

RDSON=VDSID=VDSβn[VGS-VTVDS]=1βn[VGS-VT]

RDSON=1βn[VGS-VT]

- Calculate the drain-to-source on resistance of the AOTFL296L n-channel enhancement mode power MOSFET in the Ohmic region using your previously derived expression for RDSOn along with your calculated βn value from part a.

(Hint: Your calculated RDSOn value should be between 10 and 20 mΩ.) (2 Points.)

RDSON=129.932AV2*5V-2.9V=0.01591 Ω

RDSON=15.91 mΩ

- While the AOTFL296L n-channel enhancement mode power MOSFET has impressive drain current specifications, the power dissipation of the TO-220F package will limit the continuous drain current that the transistor can conduct in many applications.

- Using the typical 45 ˚C/W thermal resistance for the TO-220F package, along with the maximum operating junction temperature of 175 ˚C for the AOTFL296L, determine the maximum continuous drain current this transistor can reliably conduct at an ambient temperature of 25 ˚C for a drain-to-source voltage VDS = 5 V. (Hint: Your calculated ID value should be between 0.5 and 1.0 A.) (3 points.)

P=IV=IDSVDS

IDVDS*45°CW=175°C-25°C

ID=175°C-25°C45°C/W *5V=0.6667 A

ID=0.6667 A

- Repeat the previous maximum continuous drain current calculation for the AOTFL296L n-channel enhancement mode MOSFET at an ambient temperature of 25 ˚C for a drain-to-source voltage VDS = 0.2 V. (3 points.)

ID=175°C-25°C45°C/W *0.2V=16.6667 A

ID=16.6667 A

- Determine the voltage VGate in the adjacent schematic, assuming an ideal op-amp and given that VT = 1.5 V and βn=200 mA/V2 for the enhancement mode MOSFET M1 operating in transistor saturation. (Hint: Your calculated VGate value should be between 6.5 and 7.5 V.) (4 Points.)

ID=βn2VGS-VT2

ID=VoutRload=4.5V51Ω=0.08824 A

VGS=2IDβn+VT=20.08824A200*10-3mAV2+1.5 V=2.439 V

VGate=VGS+Vout=2.439V+4.5V=6.939V

VGate=6.939V