Op-Amp HW (18 points)

(jas, Op-Amp HW.docx, 10/20/2025)

Include units and clearly label answers using 3 or 4 significant digits where appropriate. Show your work so that if necessary partial credit can be awarded.

- Using KCL, determine the output voltage Vout, for the adjacent circuit assuming an ideal op-amp.

(3 points.)

Vp=Vn=0

VR2=1.5 V

VR1=1.0 V

IR2=VR2R2=1.5 V1.5*103 kΩ= 1mA

IR1=1.0 V1000 Ω=1 mA

IR1+IR2=IRf=1 mA+1mA=2 mA

VRf=-IRfRf=-(2 mA)(3kΩ) = -6 V

Vout=-6 V

- For the adjacent non-inverting amplifier circuit assuming an ideal op-amp, calculate the value of Rf necessary for Vout = 5.0 V, given R1 = 10 kΩ and Vin = 2.0 V. (3 points.)

Vp=Vn=Vin=2.0 V

VR1=2.0 V

VR1=VR1R1=2.0 V10 kΩ=0.2 mA

Vout-VR1=VRf=5.0 V-2.0 V=3.0 V

Rf=VRfIRf=3.0 V0.2 mA=15 kΩ;

Rf=15 kΩ;

- Using the ideal op-amp assumptions for hand calculations, determine Vout in the adjacent circuit. (Hint: Vout should be < 0 V.) (3 points.)

Vp=Vn=2.5 V

VR1=5 V-2.5 V=2.5 V

IR1=VR1R1=2.5 mA

VRf=2.5 mA*2.0 kΩ= 5 V

VR1-V0=VRf

V0=VR1-VRf=2.5 V-5 V=-2.5 V

V0=-2.5 V

- Voltage dividers are often used to reduce larger voltages to smaller voltages that can be readily measured with microcontrollers. For this approach to work, the current through the voltage divider resistors must be the same. Yet some microcontroller analog inputs require non-negligible input current, meaning that the voltage divider output cannot be connected directly to some microcontroller analog inputs. Fortunately, a voltage follower op-amp can be used to interface between a voltage divider and microcontroller input, as the voltage follower requires negligible current from the voltage divider while providing the desired voltage and current to the microcontroller input. For the adjacent circuit assuming an ideal op-amp, calculate the voltage Vmeas resulting in Vout = 5.00 V connected to a microcontroller analog input. (Hint: Vmeas should be between 14 and 15 V.)(2 points.)

Vn-Vp=Vout=5.00 V

VR2=5.00 V

IR2=VR2R2=5.00 V13.0 kΩ=0.385 mA

IR2=IR1=0.385 mA

IR1=Vmeas-VR2R1

IR1=Vmeas-VR2R1

Vmeas=(IR1)R1+VR2 = (0.385 mA)(24.9 kΩ)+5.00 V=14.58 V

Vmeas=14.58 V

- For the adjacent circuit assuming an ideal op-amp, calculate the output voltage Vout with the 1 kΩ trim pot Rpot set so that the resistance above the wiper is 1 kΩ and the resistance below the wiper is 0 Ω. 9(Hint: Vout should be between 4 and 4.5 V.)

(3 points.)

VR2=R2R1+Rpot+R2=7.5 kΩ13 kΩ+1 kΩ+7.5 kΩ=4.186 V

VR2=Vp=Vn=4.186 V

VR2 = 4.186 V

- Using the ideal op-amp assumptions for hand calculations, determine both the minimum and maximum output voltages Vout in the adjacent circuit. (Hint: Maximum Vout should be between 6.1 and 6.3 V.) (4 Points total.)

Vout_min =  4.893 V (2 points.)

Vout_max =  6.196 V (2 points.)

Vn=Vp=2.5 V

Case 1: Wiper on Rightmost Side

VRpot+VR1=2.5 V

IR1=2.5 VR1+Rpot=2.5 V7.5 kΩ+2 kΩ=0.263 mA

IR1=0.263 mA

VR2=IR2R2=0.263 mA9.1 kΩ=2.393 V

Vout=2.5 V+2.393 V=4.893 V

Vout_min=4.893 V

Case 2: Wiper on Leftmost Side

VR1=2.5 V

IR1=2.5 V7.5 kΩ=0.333 mA

IR2+Rpot=0.333 mA

Vout=9.1 kΩ+2 kΩ0.333 mA+2.5 V

Vout=6.196 V