DC Output Power Supply Calculations Summary

(jas, DC Output Power Supply Calculations Summary.docx, 12/13/2024)

For this portion of the DC Output Power Supply Project, complete your calculations for R5, Icalc, Rbias2, Ibias2 for Vin = 21 V, R1, Rf, IR1, Cf and Input Ripple Voltage VR for ILoad = 0.9Icalc. Then replace the blue text in Table 1 with your values, which should be within the ranges given. Please reference the document “DC Output Power Supply Project.docx” for details on how to perform the following calculations. Utilize standard 5% resistor values and standard 10% capacitor values for all calculated resistor and capacitor values. Standard 5% resistors and 10% capacitor values are as follows:

- Standard 5% resistor values are limited to the following numerical values per decade of resistor values: 10, 11, 12, 13, 15, 16, 18, 20, 22, 24, 27, 30, 33, 36, 39, 43, 47, 51, 56, 62, 68, 75, 82 and 91. For example, 1.0 Ω, 10 Ω, 100 Ω, 1.0 kΩ, 10 kΩ, 100 kΩ, etc., are standard 5% values.

- Standard 10% capacitor values are limited to the following numerical values per decade of capacitor values: 10, 12, 15, 18, 22, 27, 33, 39, 47, 56, 68, and 82.

Include your calculations below for the following component values for your DC Output Power Supply: R5 and Icalc, Rbias2 and IRbias2 for Vin = 21 V, R1, Rf and IR1, Cf and Input Voltage Ripple VR for ILoad = 0.9Icalc. Points will be deducted for missing calculations.

R5 and Icalc

in1=21V

Vfiltered=in1-0.7=20.3V

VR5=0.66V

Vout=Vref=7V

VS=7V+0.66V=7.66V

VDS=20.3V-3.16V=12.64V

175°C-50°C62.5°C/W=2W

2W12.64V=0.15823A=158.23mA

Icalc=158.23mA

R5=0.66VIlimit=0.66V158.23mA=4.1711Ω≈4.3Ω

R5=4.3Ω

Iactual=0.66V4.3Ω=153.49mA

Iactual=153.49mA

Rbias2 and Ibias2 for Vin = 21 V

100μA≤Ibias≤1mA

Ibias2Chosen=0.55mA

Rbias2=33kΩ

Ibias2Actual=Vbias2Rbias2=17.8V33kΩ=0.5394mA

Vrbias2=21V-0.7V-2.5V=17.8V

Rbias2=Vbias2Ibias2=17.8V0.55mA=32.36kΩ≈33kΩ

Ibias2=0.5394mA

### R1, Rf and IR1

2.5kΩ<R1<250kΩ

10μA<IR1<1mA

IR1Chosen=0.505mA

Vref=VoutChosen=7V

Vn=Vp=2.5V

Vref-2.5VRf=2.5VR1

R1=2.5V0.505mA=4.9kΩ≈5.1kΩ

R1=5.1kΩ

Rf=7-2.5V5.1kΩ2.5V=9.18kΩ≈9.1kΩ

Rf=9.1kΩ

IR1=2.5V5.1kΩ=0.4902mA

IR1=0.4902mA

Cf and Input Ripple Voltage VR for ILoad = 0.9Icalc

Max Capacitor=1000μF

VR=1V

f=60Hz→fR=120Hz

Iload=117.86mA

VR=IloadfRCf→Cf=IloadfRVR=153.49mA120Hz*1V=1279μF

1279μF→TOO BIG!

Cf=1000μF

VRactual=IloadfRCf=153.49mA120Hz*1000μF=1.279V

VR*0.9=1.151V

90%VR=1.151V

After performing the calculations, check your values by downloading and running the calcCheck_Student.m MATLAB script. This can be done using a downloaded version of MATLAB on your desktop/laptop or using MATLAB Online. The MATLAB file calcCheck_Student.m provides a quick check of your calculations and chosen values prior to soldering components onto the PCB. To utilize this MATLAB script, first run the script and then when prompted in the Command Window enter your values for Vout, R5, Rbias2, R1, Rf and Cf followed by Enter. The MATLAB script then provides feedback information for you in the Command Window.

### Replace the Ranges of Values in the Table below with your values including units.

Table 1: Power Supply Design Chosen and Calculated Values.

| Vout | 7V |
| --- | --- |
| R5 | 4.3Ω |
| Icalc | 153.49mA |
| Rbias2 for Vin = 21 V | 33kΩ |
| IRbias2 for Vin = 21 V | 0.5394mA |
| R1 | 5.1kΩ |
| Rf | 9.1kΩ |
| IR1 | 0.4902mA |
| Cf | 1000 µF |
| Input Ripple Voltage VR for ILoad = 0.9Icalc. | 1.151V |