T-Line Homework 1 (17 Points)

(jas, T-Line HW1.docx, 4/05/2025)

Include units and clearly label answers using 3 or 4 significant digits where appropriate. Show your work so that if necessary partial credit can be awarded.

- A coaxial cable has the following parameters:

Dielectric: εr = 2.4. σ = 2 x 10-6 S/m.

Conductors: Rs = 2.6 x 10-4 Ω. σ = 5.8 x 107 S/m. μr = 1.

a = 0.2 cm. b = 0.5 cm.

Determine the following values utilizing the relationships given in Table 2-1 for R’, G’, L’ and C’:

- R’. (2 points.)

- G’. (2 points.)

- L’. (2 points.)

- C’. (2 points.)

- Calculate the characteristic impedance of the coaxial cable, assuming R’ and G’ equal zero, corresponding to a lossless transmission line. (Hint: Z0 should be between 30 Ω and 40 Ω.) (2 points.)

- A TEM wave of frequency 10 MHz propagates on a 1-meter-long transmission line with a phase (propagation) velocity up=220×106ms.

- Calculate the electrical length for this TEM wave. (Hint: The wavelength should be between 18 to 24 m.) (2 points.)

- Using a l/λ ratio threshold of 1/20 or 0.05 determine whether this transmission line is electrically short or electrically long. (1 point.)

- A TEM wave of frequency 1 MHz propagates on a lossless transmission line consisting of two parallel wires of length 6 meters surrounded by a dielectric with a permittivity ε=70.8×10-12Fm, and a permeability μ=4π×10-7 H/m, with the phase velocity given as up=1με.

- Determine the phase (propagation) velocity up in units of m/s for the TEM wave. (Hint: Your answer should range between 100 x 106 and 110 x 106 m/s). (2 points.)

- Determine whether this transmission line is electrically short or electrically long using a l/λ ratio threshold of 1/20. (2 points.)