Wave Propagation Homework (32 Points)

(jas, Wave Propagation HW.docx, 11/20/2025)

Use units and clearly label answers using 3 or 4 significant digits where appropriate. Show your work so that if necessary partial credit can be awarded.

- The adjacent uniform plane wave traveling in the +z direction has electric and magnetic field vector values of E(x, y, z =0) = xEx0 and H(x, y, z =0) = yHy0 for all values of x and y. Determine each of the following values for this uniform plane wave at z = 0. Don’t worry about including units in this problem. (4 points total.)

Ex = Ex0  (0.5 points.)

Ey = 0 (0.5 points.)

Ez = 0 (0.5 points.)

Hx = 0 (0.5 points.)

Hy = Hy0 (0.5 points.)

Hz = 0 (0.5 points.)

∂Ex∂x=0 (0.5 points.)

∂Hy∂y=0 (0.5 points.)

- An electromagnetic plane wave is traveling through pure water (εr = 81, µr = 1, σ = 0) in the +z direction with electric field given as Ez,t=x1.50cos2π1 MHzt-kz Vm. (14 points total.)

- Determine each of the following, including units: (8 points total.)

Phase Velocity     -      up = cϵr=3*108m/s81=3.33*107m/s (2 points.)

Wavelength          -        λ =upf=3.33*107m/s1*106Hz=33.3 m  (2 points.)

Wavenumber       -        k = 2πλ=2π33.3m=0.1885 rad/s  (2 points.)

Intrinsic Impedance – η = μϵ=μrμ0ϵrϵ0=14π*10-7H/m81(8.85*10-12F/m)=41.87 Ω  (2 points.)

- Using the previously determined values for  Ez,t and  η, write the complete expression for H(z, t) in terms of numeric values, including units and a unit vector. (6 points.)

Hz,t=Ex0+η=1.50 V41.84 Ω=0.0358 A

Hz,t= y0.0358cos2π1 MHzt-0.1885zA/m

- Using MATLAB, generate an m-file to produce a plot of skin depth versus frequency for a copper conductor assuming μr=1, and σ=5.8×107Sm, using the following expression for skin depth for good conductors: δs=1πfμσ. Use the logspace() command to generate a logarithmic frequency vector ranging from 1 Hz to 10 GHz, consisting of 2000 points.  Use the loglog()command to plot skin depth in millimeters (mm) on the y-axis, with scaling necessary to arrive at mm instead of meters.  Include the title shown below ending with your name, x-axis label, and y-axis label as shown in the plot below.  Replace the plot below with your version. (6 points.)

- Using MATLAB, generate an m-file to produce a plot of dB power ratio (10log10(Ratio)), and dB voltage ratio (20log10(Ratio)). Use the logspace() command to generate a logarithmic Ratio vector ranging from 10-2 to 106, consisting of 2000 points.  Use the semilogx() command to plot dB_Power ratio and dB_Voltage ratio versus Ratio.  Include the title shown below ending with your name, x-axis label, y-axis label and legend as shown in the plot below.  Replace the plot below with your version. (8 points.)