Switching Regulator Component Selection Summary

Seth Ricks

11/27/2025

ECEN 390

Switching Regulator Component Selection Summary

(jas, Switching Regulator Component Selection Summary.docx, 11/27/2025)

### For this portion of the Switching Regulator Project, complete the table below with your desired output voltage, along with the necessary calculations. Include your calculations below the table, as points will be deducted for correct values without including the necessary calculations.

- Standard 1% resistors are limited to the following numerical values per decade of resistor values:

| 10.0 | 10.2 | 10.5 | 10.7 | 11.0 | 11.3 | 11.5 | 11.8 | 12.1 | 12.4 | 12.7 | 13.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13.3 | 13.7 | 14.0 | 14.3 | 14.7 | 15.0 | 15.4 | 15.8 | 16.2 | 16.5 | 16.9 | 17.4 |
| 17.8 | 18.2 | 18.7 | 19.1 | 19.6 | 20.0 | 20.5 | 21.0 | 21.5 | 22.1 | 22.6 | 23.2 |
| 23.7 | 24.3 | 24.9 | 25.5 | 26.1 | 26.7 | 27.4 | 28.0 | 28.7 | 29.4 | 30.1 | 30.9 |
| 31.6 | 32.4 | 33.2 | 34.0 | 34.8 | 35.7 | 36.5 | 37.4 | 38.3 | 39.2 | 40.2 | 41.2 |
| 42.2 | 43.2 | 44.2 | 45.3 | 46.4 | 47.5 | 48.7 | 49.9 | 51.1 | 52.3 | 53.6 | 54.9 |
| 56.2 | 57.6 | 59.0 | 60.4 | 61.9 | 63.4 | 64.9 | 66.5 | 68.1 | 69.8 | 71.5 | 73.2 |
| 75.0 | 76.8 | 78.7 | 80.6 | 82.5 | 84.5 | 86.6 | 88.7 | 90.9 | 93.1 | 95.3 | 97.6 |

For example, 1.00 Ω, 10.0 Ω, 100 Ω, 1.00 kΩ, 10.0 kΩ, 100 kΩ, etc., are standard 1% values.

- Standard 10% capacitor values are limited to the following numerical values per decade of capacitor values: 10, 12, 15, 18, 22, 27, 33, 39, 47, 56, 68, and 82.

Include your calculations and parts selection below for the following component values for your DC power supply: R1 and R2, L1 along with Chosen Inductor, Cout along with Chosen Capacitor, ID_Avg along with Chosen Catch Diode. Points will be deducted for missing calculations.

R1 and R2

R2: Greater than 100 kΩ→1.02MΩ

R2=1.02 MΩ

R1=R2 Vout1.19V-1=1.02MΩ5.0V1.19V-1=3.27MΩ≈3.24 MΩ

R1=3.24 MΩ

L1 along with Chosen Inductor

L1=Vout+0.5V800kHz=5.0V+0.5V800kHz=6.875μH

L1=6.875μH

Chosen Inductor: SRR7045-6R8M, 6.8μH, 2A Peak Current

Cout along with Chosen Capacitor

Cout=100Voutfsw=1005.0V800kHz=25.0μF≈27.0μF

Cout=25.0μF

Chosen Capacitor:27.0μF

ID_Avg along with Chosen Catch Diode

ID_Avg=1A24V-Vout24V=1A24V-5.0V24V=0.792A

ID_Avg=0.792A

Chosen Catch Diode: 1N5818, forward current = 1A, Reverse Breakdown = 30V

### After performing the calculations, check your values by downloading and running the calcCheck_Student.m MATLAB script. This can be done using a downloaded version of MATLAB on your desktop/laptop or using MATLAB Online. The MATLAB file calcCheck_Student.m provides a quick check of your calculations and chosen values.

### Replace the Ranges of Values in the Table below with your chosen and calculated values.

Table 1: Power Supply Design Chosen and Calculated Values.

| Vout Chosen | 5.0 V |
| --- | --- |
| R2 | 1.02MΩ |
| R1 | 3.24MΩ |
| R1 + R2 | 4.26MΩ |
| L1 Calculated | 6.875μH |
| Part number, inductance value and peak current of chosen inductor L1 | SRR7045-6R8M, 6.8μH, 2A Peak Current |
| Cout Calculated | 25.0μF |
| Cout Chosen | 27.0μF |
| ID_Avg | 0.792 A |
| Part number, forward current and reverse breakdown voltage of chosen catch diode D1 | 1N5818, forward current = 1A, Reverse Breakdown = 30V |