ECEN 390 – Equation Sheet 2 (12/12/2024)

Coulombs Law: Fe21=R12q1q24πεR122. Point Charge: E =q4πεR2R.   At distance R from q.

Conducting Sphere (Surface Charge = Q): E =Q4πεR2R.   Outside at distance R from center.

Infinite Line Charge (Line Charge Density = ρl):  E =ρl2πεrr.    At distance r from line.

Infinite sheet of charge (Surface Charge Density = ρs): E =ρs2εz. z is a unit normal vector.

Permittivity: ε=εrε0 with ε0 = 8.85 x 10-12 Fm.  D = εE.

Permeability: μ=μrμ0, with μ0=4π×10-7Hm. B=μH.

Except for ferromagnetics, μr≈1 for dielectrics and conductors.

Coaxial Cable: C'=2πεlnba. L'=μ2πlnba.

Parallel-Plate Capacitor: C= QV=εrε0Ad.   W=12CV2.

Ceq_series=11C1+1C2+ ….. + 1Cn. Ceq_parallel=C1+C2+…Cn.

Inductance (Self-Inductance): L=μ0μrN2Sl (H) for a solenoid., where N is the number of

turns, S is cross sectional area, and l is solenoid length. Inductor Energy Storage W=12LI2.

Infinite straight wire carrying current I: B = μH=ϕμI2πr  At radius r from the wire.

Toroid with N turns and radius R carrying current I: B =-ϕμNI2πr   Inside Toroid.

Solenoid with N turns and length l carrying current I: B = zμNIl   Inside Solenoid.

Resistance: R=lσA Ω. Conductance: G=1R=σAl S.  Current Density: J = σE (A/m2).

Area of a circle A=πr2, where r is the radius. Electrical Power p≜dwdt. Energy:  w=t0tptdt.

Wavelength: λ=upf. Phase velocity: up=λf =1με=cεr.

For an ideal transformer, Pp=Ps.   VsVp=NsNp. IsIp=NpNs. Reflected impedance: Zin=ZLNpNs2.

Intrinsic Impedance eta: η=με.  Hz,t=Ex0+η. Complex Permittivity: εc=ε-jσω.

Skin depth: δs=1πfμσ.

For a round copper wire: RAC=l2πaπfμσ, for radius a, lenght l, and f≫0.

Power Gain GdB=10log10P1P2. Voltage Gain AdB=20log10V1V2.

Solid Angle Ω = S/r2 steradians (sr), where S is the spherical surface area subtended by a cone of radius r. The maximum solid angle is 4π sr, corresponding to a sphere.

Surface area of a sphere A=4πr2.

Antenna Directivity D equals the ratio of the pattern solid angle of an isotropic antenna, i.e., 4π, to the pattern solid angle of the given antenna. 1 ≤ D ≤ ∞.

Radiation Efficiency: ξ≜PradPtotal.Antenna Gain G=ξD, resulting in G<D for real antennas.

For a Half-Wave Dipole Antenna, total antenna length is λ/2 with Rrad≈73 Ω.

For a Quarter-Wave Monopole Antenna, total antenna length is λ/4 with Rrad≈37 Ω.