NAME: 		                                                            Score: ________/ 29

ECEN 324

Homework #9 – Chapter 3 Part 1 – Machine-Level Representation

- (9) Practice Problem 3.1

Assume the following values are stored at the indicated memory addresses and registers:

| Register | Value |
| --- | --- |
| %rax | 0x100 |
| %rcx | 0x1 |
| %rdx | 0x3 |

| Address | Value |
| --- | --- |
| 0x100 | 0xFF |
| 0x104 | 0xAB |
| 0x108 | 0x13 |
| 0x10C | 0x11 |

Fill in the following table showing the values for the indicated operands:

| Operand | Value |
| --- | --- |
| %rax | 0x100 |
| 0x104 | 0xAB |
| $0x108 | 0x108 |
| (%rax) | 0xFF |
| 4(%rax) | 0xAB |
| 9(%rax, %rdx) | 0x11 |
| 260(%rcx, %rdx) | 0x13 |
| 0xFC(, %rcx, 4) | 0xFF |
| (%rax, %rdx, 4) | 0x11 |

Assignment continues on the next page.

- (6) Practice Problem 3.6

Suppose register %rax holds value x and %rcx holds value y.  Fill in the table below with formulas indicating the value that will be stored in register %rdx for each of the following assembly code instructions.

| Expression | Result |
| --- | --- |
| leaq 6(%rax), %rdx | x+6 |
| leaq (%rax, %rcx), %rdx | x+y |
| leaq (%rax, %rcx, 4), %rdx | x+4y |
| leaq 7(%rax, %rax, 8), %rdx | 9x+7 |
| leaq 0xA( , %rcx, 4), %rdx | 4y+10 |
| leaq 9(%rax, %rcx, 2), %rdx | x+2y+9 |

- (12) Practice Problem 3.8

Assume the following values are stored at the indicated memory addresses and registers:

| Address | Value |
| --- | --- |
| 0x100 | 0xFF |
| 0x108 | 0xAB |
| 0x110 | 0x13 |
| 0x118 | 0x11 |

| Register | Value |
| --- | --- |
| %rax | 0x100 |
| %rcx | 0x1 |
| %rdx | 0x3 |

Fill in the following table showing the effects of the following instructions, both in terms of the register or memory location that will be updated and the resulting value:

| Instruction | Destination (memory addr. or register name) | Value |
| --- | --- | --- |
| addq %rcx, (%rax) | 0x100 | 0xFF+0x1=0x100 |
| subq %rdx, 8(%rax) | 0x100+8=0x108 | 0xAB-0x3=0xAB |
| imulq $16, (%rax, %rdx, 8) | 0x100 + 0x3*8=0x118 | 0x11*16=0x110 |
| incq 16(%rax) | 0x100+16=0x110 | 0x13+1=0x14 |
| decq %rcx | %rcx | 0x1-1=0x0 |
| subq %rdx, %rax | %rax | 0x100-0x3=0xFD |

- (2) Similar to a subset of Practice Problem 3.11

In the compilation of the loop

for (i = 0; i < n; i++)

v += i;

we find the following assembly code line:

xorq %rdx, %rdx

Explain why this instruction would be there, even though there are no EXCLUSIVE-OR operators in our C code.  What operation in the C program does this instruction implement?

This line of code sets the value stored in the register %rdx to 0. I would say that this is the variable i, initialized to zero for the loop. v and n were declared previously, and so probably took the first and second argument registers. The third, %rdx, appears to be used for the index.