NAME: __________Seth Ricks__________	                                                            Score: ________/ 18

ECEN 324

Homework #12 – Chapter 5 – Optimizing Program Performance

- (3) Practice Problem 5.1

The following problem illustrates the way memory aliasing can cause unexpected program behavior.  Consider the following procedure to swap two values:

/* Swap value x at xp with value y at yp */

void swap (long *xp, long *yp)

{

*xp = *xp + *yp; /* x+y */

*yp = *xp - *yp; /* x+y-y = x */

*xp = *xp - *yp; /* x+y-x = y */

}

If this procedure is called with xp equal to yp, what effect will it have?

If xp is equal to yp, then the first instruction will be x = x+x, the second will be x = x -x, and the third will be x = x – x. So x = 2x, then x = 0, then x = 0. So both xp and yp will be 0.

- (6) Practice Problem 5.2

Later in this chapter we will take a single function and generate many different variants that preserve the function’s behavior, but with different performance characteristics. For three of these variants, we found that the run times (in clock cycles) can be approximated by the following functions:

Version 1	60 + 35n.		Fastest for:  0≤n≤2

Version 2	136 + 4n.		Fastest for:  3≤n≤7

Version 3	157 + 1.25n.		Fastest for:  n≥8

For what values of n would each version be the fastest of the three?  Remember that n will always be an integer.

- (5) Matching Definitions

| _B__Optimization blockers _C__Side effect __A_Code motion __E_Strength reduction _D__Memory Aliasing | - Improving the efficiency of code by rearranging where computations are done within the code. - Items that limit the ability of a compiler to perform optimizations. Examples are memory aliasing and procedural side effects. - Modifies some part of the global program state in addition to performing the requested operation. - When data in memory can be accessed by different names, such as having two pointers point to the same memory location. - Replacing costly operations with simpler ones. One example is replacing multiplication with shifts and adds. |
| --- | --- |

- (4) Part of Practice Problem 5.3

Consider the following functions:

long min(long x, long y) { return x < y ? x : y; }

long max(long x, long y) { return x < y ? y : x; }

void incr(long *xp, long v) { *xp += v; }

long square(long x) { return x * x; }

The following two code fragments call these functions:

A.	for (i = min(x,y); i < max(x,y); incr(&i, 1))

t += square(i);

C.	long low = min(x,y);

long high = max(x,y);

for (i = low; i < high; incr(&i, 1))

t += square(i);

Assume x equals 10 and y equals 100. Fill in the following table indicating the number of times each of the four functions are called in the code fragments A and C

| Code | min | max | incr | square |
| --- | --- | --- | --- | --- |
| A. | 1 | 91 | 90 | 90 |
| C. | 1 | 1 | 90 | 90 |