NAME: Seth Ricks		                                                             Score: ________/ 19

ECEN 324

Homework #13 – Chapter 4 – Processor Architecture

- (16) Practice Problem 4.28

Suppose we analyze the combinational logic of the following un-pipelined hardware

| 300 ps |  |  |  | 20 ps |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Combinational logic |  |  |  |  |  | R e g |  |  |  | Delay = 320 ps Throughput = 3.12 GIPS |

Clock

and determine that it can be separated into a sequence of six blocks, named A to F, having delays of 80, 30, 60, 50, 70, and 10 ps, respectively, illustrated as follows:

| 80 ps | 30 ps | 60 ps | 50 ps | 70 ps | 10 ps | 20 ps |
| --- | --- | --- | --- | --- | --- | --- |

| A |  | B |  | C |  | D |  | E |  | F |  | Reg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

We can create pipelined versions of this design by inserting pipeline registers between pairs of these blocks.  Different combinations of pipeline depth (how many stages) and maximum throughput arise, depending on where we insert the pipeline registers.  Assume that a pipeline register has a delay of 20 ps.

  - (3) Inserting a single register gives a two-stage pipeline.  Where should the register be inserted to maximize throughput?  What would be the throughput and latency?

| - 80 ps | 30 ps | 60 ps | 50 ps | 70 ps | 10 ps | 20 ps |
| --- | --- | --- | --- | --- | --- | --- |

| A |  | B |  | C | x | D |  | E |  | F |  | Reg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

To maximize throughput, we want to minimize the clock period. What is the smallest that the clock period can be? Clock period is 20ps + largest combinational logic. So we want the largest combinational logic to be the smallest possible and close to the other. This come from:

Insert the register between stages:  C & D

Clock is 80+30+60+20 ps =190ps=>1190 ps  = 5.26 GIPS

Throughput: 5.26 GIPS		Latency: 190 ps * 2 = 380 ps

  - (4) Where should two registers be inserted to maximize the throughput of a three-stage pipeline?  What would be the throughput and latency?

| 80 ps | 30 ps | 60 ps | 50 ps | 70 ps | 10 ps | 20 ps |
| --- | --- | --- | --- | --- | --- | --- |

| A |  | B | x | C |  | D | x | E |  | F |  | Reg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The combination that results in the largest combinational logic being the smallest is:

Insert the register between stages:  B & C;   D & E

Clock =80+30+20 ps=130 ps => throughput =1130ps= 7.69 GIPS

Latency = 130 ps * 3 = 390 ps

Throughput: 7.69 GIPS		Latency: ___390 ps_______

Practice Problem 4.28 continued

| 80 ps | 30 ps | 60 ps | 50 ps | 70 ps | 10 ps | 20 ps |
| --- | --- | --- | --- | --- | --- | --- |

| A | x | B |  | C | x | D | x | E |  | F |  | Reg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - (5) Where should three registers be inserted to maximize the throughput of a four-stage pipeline?  What would be the throughput and latency?

With the configuration above, the largest combinational logic is 90 ps.

Between stages:  A & B;   C & D;   D & E

Clock = 30 + 60 + 20 = 110 ps

Throughput = 1110 ps=9.09 GIPS

Throughput: 9.09 GIPS		Latency: 110 * 4 = 440 ps

  - (4) What is the minimum number of stages that would yield a design with the maximum achievable throughput?  Describe this design, its throughput, and its latency.

| 80 ps | 30 ps | 60 ps | 50 ps | 70 ps | 10 ps | 20 ps |
| --- | --- | --- | --- | --- | --- | --- |

| A | x | B | x | C | x | D | x | E | x | F |  | Reg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The most we can do is 6 stages, the largest combinational logic is 80 ps. The fastest the clock could be is 80 + 20 = 100 ps. The throughput would be =1100 ps=10 GIPS.

| 80 ps | 30 ps | 60 ps | 50 ps | 70 ps | 10 ps | 20 ps |
| --- | --- | --- | --- | --- | --- | --- |

| A | x | B | x | C | x | D | x | E |  | F |  | Reg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

But with 5 stages strategically placed, the largest combinational logic would still be 80 ps.

Throughput: 10 GIPS		Latency: 100 ps * 6 = 600 ps

(5 stages)

- (3) Subset of Practice Problem 4.29

| 300 ps |  |  |  | 20 ps |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Combinational logic |  |  |  |  |  | R e g |  |  |  | Delay = 320 ps Throughput = 3.12 GIPS |

Suppose we could take the system shown above (Figure 4.32) and divide it into an arbitrary number of pipeline stages, all having the same delay.  What would be the ultimate limit on the throughput, given pipeline register delays of 20 ps?

The limit on the throughput is the register delay. Even if we were able to divide the combinational logic into infinitesimally small pieces, so that each stage is 0, each stage would still have a 20 ps delay. The theoretical minimum throughput (which is mathmatically impossible), would be 120 ps=50 GIPS