NAME: Seth Ricks		                                                             Score: ________/ 24

ECEN 324

Homework #14 – Chapter 5 – Optimizing Program Performance

- (5) Matching Definitions

| _C__ Loop unrolling _E__k x k loop unrolling _D__ Spilling _B__ Superscalar _A__Speculative Execution | - A technique in which the processor begins fetching and decoding instructions at where it predicts the branch will go, and even begins executing these operations before it has been determined whether or not the prediction was correct. - A type of processor which can achieve a CPI less than 1.0 by fetching, decoding, and executing multiple instructions in parallel. - Reducing the number of iterations for a loop by increasing the number of elements computed on each iteration. - Storing temporary values on the stack when the degree of parallelism exceeds the number of available registers. - Using multiple accumulators (temporary variables/registers) in one iteration of a loop which is computing multiple elements in that one loop iteration. Assignment continues on the next page. |
| --- | --- |

- (5) Practice Problem 5.5

Suppose we wish to write a function to evaluate a polynomial, where a polynomial of degree n is defined to have a set of coefficients a0, a1,a2, . . . an. For a value of x, we evaluate the polynomial by computing

a0+a1x+a2x2+ ∙ ∙ ∙ + anxn

This evaluation can be implemented by the following function, having as arguments an array of coefficients a, a value x, and the polynomial degree (the value n in the equation above). In this function, we compute both the successive terms of the equation and the successive powers of x within a single loop:

1	double poly(double a [], double x, long degree)

2	{

3		long i;

4		double result = a[0];

5		double xpwr = x; /* Equals x^i at start of loop */

6		for (i = 1; i <= degree; i++) {

7		    result += a[i] * xpwr;

8		    xpwr = x * xpwr;

9	      }

10		return results;

11    }

- (2) For degree n, how many additions and how many multiplications does this code perform?  Express the answer in terms of the degree n of the polynomial and don’t include the addition done as part of the loop control (i++).

7		    result = result + (a[i] * xpwr); // one add, one mult

8		    xpwr = x * xpwr;			 // one mult

If degree is 3, then the above will happen 1, 2, 3 times.

So the number additions is n, and the number of multiplications is 2n.

- (3) On our reference machine, with arithmetic operations having the latencies shown in Figures 5.12, we measure the CPE for this function to be 5.00. Explain how this CPE arises based on the data dependencies formed between iterations due to the operations implementing lines 7-8 of the function.

7		    result = result + (a[i] * xpwr); // The multiply has to happen before the addition

8		    xpwr = x * xpwr; // This has to be done before the next cycle can do it

From figure 5.12, the latency of multiplication is 5 with a float. The next iterations of multiplications rely on the current one, so they can’t be started before this one is done. That is why the CPE is 5.

- (5) Practice Problem 5.9

The traditional implementation of the merge step of mergesort requires three loops [98]:

1	void merge(long src1[], long src2[], long dest[], long n)  {

2		long i1 = 0;

3		long i2 = 0;

4		long id = 0;

5		while (i1 < n && i2 < n) {

6			if (src1[i1] < src2[i2])

7		    		dest[id++] = src1[i1++];

8			else

9	      		dest[id++] = src2[i2++];

10		}

11		while (i1 < n)

12			dest[id++] = src1[i1++];

13		while (i2 < n)

14			dest[id++] = src2[i2++];

15    }

The branches caused by comparing variables i1 and i2 to n have good prediction performance – the only mispredictions occur when they first become false. The comparison between values src1[i1] and src2[i2] (line 6), on the other hand, is highly unpredictable for typical data. This comparison controls a conditional branch, yielding a CPE (where the number of elements is 2n) of around 15.0 when run on random data.

Rewrite the code so that the effect of the conditional statement in the first loop (lines 6-9) can be implemented with a conditional move.

6			if (src1[i1] < src2[i2])

7		    		dest[id++] = src1[i1++];

8			else

9	      		dest[id++] = src2[i2++];

int move_src1 = (src1[i1] < src2[i2]); // store the condition value

dest[id++]= (move_src1) ? src1[i1]: src2[i2]; //conditional move

i1 += move_src1; // possibly increment index 1

i2 += !move_src1; // possibly increment index 2

- (9) Homework Problems 5.13

Suppose we wish to write a procedure that computes the inner product of two vectors u and v. An abstract version of the function has a CPE of 14-18 with x86-64 for different types of integer and floating-point data. By doing the same sort of transformations we did to transform the abstract program combine1 into the more efficient combine4, we get the following code:

/* Inner product. Accumulate in temporary */

void inner4(vec_ptr u, vec_ptr v, data_t *dest)

{

long I;

long length = vec_length(u);

data_t *udata = get_vec_start(u);

data_t *vdata = get_vec_start(v);

data_t sum = (data_t) 0;

for (i=0; i < length; i++) {

sum = sum + udata[i] * vdata[i];

}

*dest = sum;

}

Our measurements show that this function has CPEs of 1.50 for integer data and 3.00 for floating-point data. For data type double, the x86-64 assembly code for the inner loop is as follows:

Inner loop of inner4. data_t = double, OP = *

udata in %rbp, vdata in %rax, sum in %xmm0

i in %rcx, length in %rbx

.L15:                                    loop:

vmovsd    0(%rbp,%rcx,8), %xmm1            Get udata[i]

vmulsd    (%rax,%rcx,8), %xmm1, %xmm1      Multiply by vdata[i]

vaddsd    %xmm1, %xmm0, %xmm0              Add to sum

addq      $1, %rcx                         Increment i

cmpq      %rbx, %rcx                       Compare i:length

jne       .L15                             If !=, goto loop

Assume that the functional units have the characteristics listed here, also Figure 5.12.

|  | Integer |  |  | Floating Point |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operation | Latency | Issue | Capacity | Latency | Issue | Capacity |
| Addition | 1 | 1 | 4 | 3 | 1 | 1 |
| Multiplication | 3 | 1 | 1 | 5 | 1 | 2 |
| Division | 3-30 | 3-30 | 1 | 3-15 | 3-15 | 1 |

Homework Problems 5.13 continued on the next page

- (4) Diagram how this instruction sequence would be decoded into operations and show how the data dependencies between them would create a critical path of operations, in the style of Figure 5.14.

- (1.5) For data type double, what lower bound on the CPE is determined by the critical path?

3, because of the left loop. The bottleneck is because the sum requires sum for the next addition.

- (1.5) Assuming a similar instruction sequence for the integer code as well, what lower bound on the CPE is determined by the critical path for integer data?

1.5. This is because we don’t have enough hardware to do all the additions. We have 6 additions and only 4 capacity. So 6 /4 = 1/5.

- (2) Explain how the floating-point versions can have CPEs of 3.00, even though the multiplication operation requires 5 clock cycles.

The hardware can do multiple operations at once, including the multiplications.